Single-Rate Table Parameter Settings

Parameter Settings (Prepaid Parameters):

Note: Configure the prepaid parameter data block, which includes the prepaid amount parameter.

Function Code: 03

Identifier: 070115FF

data field

| data item | data format | Byte length | span | Unit Type | remarks |
| --- | --- | --- | --- | --- | --- |
| ID |  | 4 |  |  | 070115FF |
| operation code |  | 4 |  |  | Fixed: 11223344 |
| Paid Model | XX | 1 | 00-01 |  | 00H: Postpaid (Default), 01H: Prepaid |
| Rate Structure | XX | 1 | 00/01 |  | 00 Single Rate / 01 Multiple Rates |
| Alarm Amount Threshold 1 | XXXXXX.XX | 4 |  | Yuan(BCD) |  |
| Alarm Amount Threshold 2 | XXXXXX.XX | 4 |  | Yuan(BCD) |  |
| limit of overdrawn account | XXXXXX.XX | 4 |  | Yuan(BCD |  |
| Accumulation Amount Limit | XXXXXX.XX | 4 |  | Yuan(BCD) | Default: ¥799,999.00 |
| Electricity tariff switching time | YYMMDDHHMM | 5 |  |  | Date and Time |
| Current usage-based tariff: 1 | XXXX.XXXX | 4 |  | Yuan(BCD) |  |
| Current usage rate: 2 | XXXX.XXXX | 4 |  | Yuan(BCD) |  |
| Current usage rate: 3 | XXXX.XXXX | 4 |  | Yuan(BCD) |  |
| Current usage rate: 4 | XXXX.XXXX | 4 |  | Yuan(BCD) |  |
| Alternative Rate Tariff 1 | XXXX.XXXX | 4 |  | Yuan(BCD) |  |
| Alternative Rate Tariff 1 | XXXX.XXXX | 4 |  | Yuan(BCD) |  |
| Alternative Rate Tariff 1 | XXXX.XXXX | 4 |  | Yuan(BCD) |  |
| Alternative Rate Tariff 1 | XXXX.XXXX | 4 |  | Yuan(BCD) |  |
| Power outage initiation time due to unpaid bills | HHMM | 2 |  | BCD | Time Division |
| Arrears settlement deadline | HHMM | 2 |  | BCD | Time Division |
| Monthly Settlement Date | DDHH | 2 |  |  | (Default: 00:00 UTC+1) |
| Daily freezing time | HHmm | 2 |  |  | (Default: 0:00) |
|  |  | 71 |  |  |  |

Sample reading:

TX：fe fe fe fe 68 07 00 51 05 03 26 68 11 04 32 48 34 3a 53 16

RX：fe fe fe fe 68 07 00 51 05 03 26 68 91 43 32 48 34 3a 33 33 33 53 33 33 33 43 33 33 33 33 33 33 cc cc cc cc 33 33 33 33 33 33 33 73 33 33 33 73 33 33 33 73 33 33 33 73 33 33 33 33 33 33 33 33 33 33 33 33 33 33 33 33 33 33 33 33 33 33 34 33 33 34 16

Example setup:

TX：fe fe fe fe 68 07 00 51 05 03 26 68 03 47 32 48 34 3a 77 66 55 44 33 33 33 53 33 33 33 43 33 33 33 33 33 33 cc cc cc cc 33 33 33 33 33 33 33 73 33 33 33 73 33 33 33 73 33 33 33 73 33 33 33 33 33 33 33 33 33 33 33 33 33 33 33 33 33 33 33 33 33 33 34 33 33 20 16

RX：fe fe fe fe 68 07 00 51 05 03 26 68 83 00 d9 16